Self-Healing

Fault-Tolerant Embedded System

Project Technical Report

Embedded Systems Design

Platform: PIC16F887 | Proteus 8 Professional | MPLAB X IDE

Team Members:

Sharvesh 24BEC057 | Siva 24BEC058 | Sohan Kolli 24BEC059 | Varshith 24BEC011

# 1. Project Overview

This project implements a fault-tolerant embedded monitoring system that continuously reads temperature data over an I²C bus and automatically detects, logs, and recovers from bus faults. The system is designed to demonstrate robust embedded system design principles, where communication failures are detected, recorded with timestamps, and automatically recovered without human intervention.

The implementation showcases real-world resilience techniques used in industrial and safety-critical applications, where sensor failures cannot be allowed to crash the entire system. Instead, faults are classified, recorded persistently, and the bus is healed automatically so normal operation resumes.

| Component | Details |
| --- | --- |
| Microcontroller | PIC16F887 (8-bit, 8MHz internal oscillator) |
| Simulation Environment | Proteus 8 Professional |
| Firmware IDE | MPLAB X IDE with XC8 Compiler |
| Communication Protocol | I²C (Inter-Integrated Circuit) |
| UART Telemetry | 9600 baud, 8N1 to Virtual Terminal / COM port |

Fig: Schematic Layout in Proteus

# 2. Project Objectives

The key objectives of this project are:

- Implement reliable I²C communication between the PIC16F887 and multiple peripherals

- Detect I²C communication and Watchdog timer reset failures in real time using hardware status registers and bus line monitoring

- Automatically recover the I²C bus and Watchdog timer using the NXP-recommended clock pulse recovery procedure

- Log every detected fault persistently to external EEPROM with a real-time timestamp

- Provide real-time UART telemetry of faults to an external monitoring interface and verification using virtual terminal in proteus temporarily

- Demonstrate self-healing behaviour where the system resumes normal operation after fault recovery

- Support fault history browsing and log management via push button interface

- Implement a dual-buffer strategy using internal PIC EEPROM to prevent data loss during bus failure

# 3. Hardware Components

| Component | Model | Purpose | Interface |
| --- | --- | --- | --- |
| Microcontroller | PIC16F887 | Main controller, firmware execution | GPIO, I²C, UART |
| Temperature Sensor | LM75AD | Continuous temperature measurement | I²C (0x90/0x91) |
| Real Time Clock | DS1307 | Fault timestamp generation | I²C (0xD0/0xD1) |
| External EEPROM | 24LC256 | Persistent fault log storage (256 records) | I²C (0xA0/0xA1) |
| LCD Display | HD44780 (LM041L) | Real-time status and temperature display | 8-bit parallel (PORTD) |
| Pull-up Resistors | 4.7kΩ | I²C bus biasing (SDA and SCL) | Passive |
| Push Buttons | Tactile | Fault injection and log browsing | GPIO (RB0, RB1) |
| Crystal | 32.768 kHz | DS1307 RTC clock source | Oscillator |

## 3.1 PIC16F887 Pin Assignment

| Pin | Function | Connected To |
| --- | --- | --- |
| RC3 (SCL) | I²C Clock | LM75, DS1307, 24LC256 SCL lines |
| RC4 (SDA) | I²C Data | LM75, DS1307, 24LC256 SDA lines |
| RC6 (TX) | UART Transmit | Virtual Terminal / COMPIM RXD |
| RC7 (RX) | UART Receive | Not used (receive disabled) |
| RD0–RD7 | LCD Data Bus | HD44780 data pins |
| RE0 (RS) | LCD Register Select | HD44780 RS pin |
| RE1 (E) | LCD Enable | HD44780 Enable pin |
| RB0 | Button 1 | Log clear / fault injection |
| RB1 | Button 2 | Fault log browsing |

# 4. System Architecture

The system is organised into five independent firmware modules, each responsible for a distinct aspect of system behaviour. All modules interact through shared state variables and the I²C bus.

## 4.1 Module Overview

| Module | Responsibility |
| --- | --- |
| Sensor Monitoring | Continuous LM75 temperature reading and LCD display |
| Fault Detection | Bus line monitoring, ACKSTAT checking, timeout detection |
| Bus Recovery | NXP I²C 9-pulse clock recovery and MSSP reinitialization |
| Fault Logging | Dual-buffer EEPROM write with timestamp from DS1307 |
| UART Telemetry | Real-time fault transmission at 9600 baud |

## 4.2 Data Flow

The following sequence describes the complete system data flow during normal and fault conditions:

Temperature Sensor (LM75)

|

v

PIC reads temperature via I²C

|

[Success]        [Failure]

|                 |

Display on LCD    Classify Fault

|

Get timestamp (RTC)

|

Buffer in internal EEPROM

|

Send UART telemetry

|

I²C Bus Recovery

|

Flush to external EEPROM

|

Resume normal operation

# 5. Fault Detection and Classification

The system detects five distinct fault types. Each fault type is identified using a combination of hardware register inspection and I/O pin state checking. The firmware uses a structured fault context to record which step of the I²C transaction failed and whether an ACKSTAT bit was set, enabling accurate classification after the retry sequence exhausts all attempts.

## 5.1 SDA Stuck Fault

The SDA (Serial Data) line carries all data bits in an I²C transaction. In the idle state both SDA and SCL must remain HIGH. If a slave device holds SDA LOW permanently, the master cannot initiate a new transaction because the bus appears busy.

Common causes in real systems:

- Slave device crashes mid-transfer, holding SDA low waiting for further clock pulses

- Power glitch interrupts a byte transfer, leaving the slave expecting more clocks

- Hardware failure in the slave driver circuit

Detection method:

if (PORTCbits.RC4 == 0)  ->  FAULT_SDA_STUCK

Recovery procedure:

- MSSP module is disabled (SSPEN = 0)

- SCL is toggled manually 9 times, providing up to 9 clock pulses

- After 9 pulses the slave has received enough clocks to complete any pending byte

- A manual STOP condition is generated

- MSSP is reinitialized with original configuration

- SDA line is checked — if still LOW, procedure repeats

## 5.2 SCL Stuck Fault

The SCL line carries the clock signal. If it is held LOW, the master cannot generate any clock pulses. The I²C protocol allows slaves to perform clock stretching (holding SCL LOW briefly), but if a slave never releases SCL the bus freezes permanently.

Common causes:

- Slave performing indefinite clock stretching

- Bus contention between two masters

- Slave firmware crash during a transaction

Detection method:

if (PORTCbits.RC3 == 0)  ->  FAULT_SCL_STUCK

Recovery procedure:

Same as SDA stuck recovery. The 9-pulse clock sequence forces the slave to release SCL by completing whatever operation it was waiting on.

## 5.3 NACK Fault

After every byte transmitted on I²C, the receiver must acknowledge receipt by pulling SDA LOW during the 9th clock pulse. If SDA remains HIGH during this period, it is a Not-Acknowledge (NACK). A NACK indicates the addressed device either does not exist on the bus or cannot accept the data.

Common causes:

- Device address incorrect

- Sensor powered off or disconnected

- Device busy and unable to accept commands

- Register address out of range

Detection method:

if (ACKSTAT)  ->  FAULT_NACK

ACKSTAT is a bit in the SSPCON2 register that is set when a NACK is received from the slave.

Recovery:

The system retries up to 3 times before declaring the fault. After logging, recovery follows the standard bus reset procedure. If the device is reconnected and the bus recovers, normal operation resumes automatically.

## 5.4 Timeout Fault

A timeout occurs when an I²C operation fails to complete within a defined number of software loop iterations. Without timeout protection, the MCU would block indefinitely waiting for a hardware flag that never arrives, causing a complete system freeze.

Detection method:

#define TIMEOUT 6000

timeout_counter = TIMEOUT;

while (SSPIF == 0) {

if (--timeout_counter == 0) return 0;  // FAULT_TIMEOUT

}

The timeout counter is used in every blocking wait in the I²C driver: wait_i2c_idle(), wait_sspif_and_clear(), start(), stop(), and read_byte(). Any of these failing to complete within 6000 iterations returns failure.

Fault injection in Proteus:

Holding SDA or SCL LOW during a transaction prevents the MSSP hardware from completing, causing the timeout counter to expire.

## 5.5 Watchdog Reset Fault

The PIC16F887 incorporates a hardware Watchdog Timer (WDT). The firmware must periodically execute CLRWDT (Clear Watchdog Timer) to prevent the WDT from expiring. If the firmware hangs in an infinite loop or deadlock, the WDT resets the MCU automatically.

Detection method:

if ((PCON & 0x10) == 0)  ->  FAULT_WATCHDOG

The PCON register contains the NOT_TO bit (bit 4). After a watchdog reset this bit reads 0, distinguishing it from a normal power-on reset where it reads 1.

The system logs this fault on every startup where a watchdog reset is detected, using the last available RTC timestamp. This ensures even unexpected resets leave a trace in the fault log.

| Fault | Cause | Detection Register/Pin | Proteus Injection | Recovery |
| --- | --- | --- | --- | --- |
| SDA STUCK | Slave holds data line LOW | RC4 == 0 | Button pulls SDA to GND | 9-pulse SCL + STOP + reset |
| SCL STUCK | Clock line held LOW | RC3 == 0 | Button pulls SCL to GND | 9-pulse SCL + STOP + reset |
| NACK | Device not responding | ACKSTAT == 1 | Remove/disable LM75 | 3 retries then bus reset |
| TIMEOUT | Bus operation never completes | Counter expires (6000) | Hold SDA/SCL LOW | I²C module reinit |
| WATCHDOG | Firmware freeze | PCON bit 4 == 0 | Infinite loop injection | MCU auto-reset + log |

# 6. Self-Healing Bus Recovery

The bus recovery procedure follows the NXP I²C specification recommendation for recovering a stuck bus. It is implemented in the I2C_bus_recovery() function and is called automatically every time a fault is detected after the retry sequence fails.

## 6.1 Recovery Sequence

Step 1: Disable MSSP module (SSPEN = 0)

Step 2: Configure RC3 (SCL) as GPIO output

Step 3: Configure RC4 (SDA) as GPIO input

Step 4: Toggle SCL HIGH/LOW up to 9 times

Check SDA after each pulse

Stop early if SDA goes HIGH (slave released)

Step 5: Drive SCL HIGH

Step 6: Generate manual STOP condition:

SDA LOW while SCL HIGH

SCL HIGH, then SDA HIGH

Step 7: Call I2C_reset() to reinitialize MSSP

Step 8: Restore RC3, RC4 as I²C pins (TRISC3=1, TRISC4=1)

## 6.2 Why 9 Clock Pulses?

The I²C protocol transfers data in 8-bit bytes followed by 1 acknowledge bit, totalling 9 clock cycles per byte. A slave stuck mid-transfer may be waiting for up to 8 more clock pulses to complete the current byte before it can release SDA. Providing exactly 9 pulses guarantees the slave can complete any in-progress byte, regardless of where in the sequence it got stuck.

## 6.3 MSSP Reinitialization

SSPCON  = 0x28;   // I²C master mode, SSPEN=1

SSPSTAT = 0x80;   // Slew rate control disabled

SSPCON2 = 0x00;   // Clear all control bits

SSPIF   = 0;      // Clear interrupt flag

# 7. Fault Logging System

The system uses a dual-buffer logging strategy. When a fault occurs, the I²C bus may be non-functional, making it impossible to write directly to the external EEPROM. The firmware therefore first buffers the fault in the PIC's own internal EEPROM (which is always accessible), then transfers it to external EEPROM once the bus is recovered.

## 7.1 Internal EEPROM Buffer

The PIC16F887 has 256 bytes of internal data EEPROM. The system reserves the first 10 bytes for fault buffering and state persistence:

| Address | Content | Purpose |
| --- | --- | --- |
| 0x00 | Fault type byte | Type of fault (1–5) |
| 0x01 | Hour | RTC hour at fault time |
| 0x02 | Minute | RTC minute at fault time |
| 0x03 | Second | RTC second at fault time |
| 0x04 | Valid flag | 1 = unprocessed fault waiting, 0 = empty |
| 0x05 | Write index low byte | Current EEPROM write position (low) |
| 0x06 | Write index high byte | Current EEPROM write position (high) |
| 0x07 | Log count low byte | Total fault records logged (low) |
| 0x08 | Log count high byte | Total fault records logged (high) |
| 0x09 | Magic byte (0xA5) | First-run detection sentinel |

## 7.2 External EEPROM Layout (24LC256)

Each fault record occupies exactly 4 bytes in the external 24LC256 EEPROM. With 32KB capacity, the system supports up to 256 fault records before wrapping:

| EEPROM Address | Content | Example Value |
| --- | --- | --- |
| 0x0000 | Fault type | 0x02 (SCL STUCK) |
| 0x0001 | Hour (BCD decoded) | 0x12 (12h) |
| 0x0002 | Minute (BCD decoded) | 0x07 (07m) |
| 0x0003 | Second (BCD decoded) | 0x14 (20s) |
| 0x0004 | Next record fault type | 0x01 (SDA STUCK) |
| 0x0005–0x0007 | Next record timestamp | ... |

## 7.3 First-Run Detection

On first power-up, the internal EEPROM contains 0xFF in all locations. The firmware checks for the magic byte 0xA5 at address 0x09. If absent, it initialises write_index and log_count to zero and writes the magic byte, preventing ghost data from blank EEPROM being interpreted as valid log state.

if (internal_eeprom_read(INTERNAL_EEPROM_INIT_ADDR) != 0xA5)

{

write_index = 0;

log_count   = 0;

save_log_state();   // Also writes magic byte

}

# 8. UART Telemetry

Every fault event generates a UART message transmitted over RC6 (TX) at 9600 baud. This allows real-time monitoring of fault events on a connected terminal without needing to interact with the LCD or button interface.

## 8.1 UART Configuration

| Parameter | Value |
| --- | --- |
| Baud Rate | 9600 |
| Data Bits | 8 |
| Parity | None |
| Stop Bits | 1 |
| SPBRG Register | 51 (for 8MHz, BRGH=1) |
| Mode | Asynchronous transmit only (CREN=0) |

## 8.2 Message Format

Each UART fault message follows a fixed format:

Fault:<TYPE> Time:<HH>:<MM>:<SS>\r\n

Example messages:

Fault:SDA STUCK Time:12:07:14

Fault:SCL STUCK Time:12:08:02

Fault:NACK Time:12:09:55

Fault:TIMEOUT Time:12:10:33

Fault:WD RST Time:12:11:00

## 8.3 Duplicate Prevention

The firmware uses a RAM flag (fault_uart_sent) to ensure each unique fault event generates exactly one UART message, even if the fault condition persists across multiple main loop iterations. The flag is reset when the fault is successfully flushed to external EEPROM.

# 9. Firmware Structure

The firmware is written in C for the XC8 compiler and structured into clearly separated functional modules. All modules are contained in a single translation unit (esd1.c) for simplicity in the Proteus simulation environment.

| Function / Module | Description |
| --- | --- |
| uart_init() | Configures USART for 9600 baud async TX, sets RC6 idle HIGH |
| uart_tx_char() | Blocks until TXIF set, then writes character to TXREG |
| uart_send_fault_log() | Formats and transmits complete fault message string |
| wait_i2c_idle() | Polls SSPCON2/SSPSTAT with timeout protection |
| start() / stop() | Generates I²C START/STOP conditions with timeout |
| address() / write() | Transmits address/data byte, checks ACKSTAT |
| read_byte() | Receives byte, sends ACK or NACK |
| I2C_bus_recovery() | Full NXP 9-pulse recovery procedure |
| I2C_reset() | Reinitialises MSSP peripheral registers |
| rtc_read() | Reads HH:MM:SS from DS1307 via I²C |
| eeprom_write_byte() | Writes single byte to 24LC256 at 16-bit address |
| eeprom_read_byte() | Reads single byte from 24LC256 |
| internal_eeprom_write() | Writes to PIC internal EEPROM with required unlock sequence |
| buffer_fault_in_internal_eeprom() | Safely stores fault in internal EEPROM if no pending fault |
| flush_internal_eeprom_fault() | Transfers buffered fault to external EEPROM after recovery |
| log_fault() | Writes 4-byte fault record to external EEPROM, updates counters |
| classify_fault() | Determines fault type from context and bus pin states |
| read_temp_with_classification() | Reads LM75 with per-step fault context recording |
| restore_log_state() | Loads write_index and log_count from internal EEPROM on boot |
| save_log_state() | Persists write_index, log_count, and magic byte to internal EEPROM |
| clear_external_eeprom() | Resets log counters (soft clear, triggered by RB1) |
| display_fault_record() | Reads and displays a fault record on LCD by index |

## 9.1 Initialization Sequence

The correct initialization order is critical. UART must be initialized before I²C to prevent the SSP peripheral from interfering with RC6 before the USART claims it:

1. Configure port directions (TRISA–TRISE)

2. Disable analog inputs (ANSEL = 0)

3. Set oscillator to 8MHz (OSCCON = 0x71)

4. Initialize UART (uart_init) ← must come before I²C

5. Initialize MSSP for I²C master (SSPCON = 0x28)

6. Initialize LCD

7. Perform I²C bus recovery

8. Restore log state from internal EEPROM

9. Check PCON for watchdog reset, log if found

10. Enter main monitoring loop

# 10. System Operation

## 10.1 Normal Operation

During normal operation the main loop executes the following sequence every approximately 1 second:

- Read temperature from LM75 (up to 3 retry attempts)

- If successful: flush any pending internal EEPROM fault, update last known good RTC timestamp, display temperature and SYSTEM OK on LCD

- If all retries fail: classify fault, send UART telemetry, buffer in internal EEPROM, perform bus recovery, display fault status on LCD for 2 seconds

- Check RB1 button for log clear request

- Clear watchdog timer (CLRWDT)

## 10.2 LCD Display

The LCD shows two lines of information:

| Condition | Line 1 (0x80) | Line 2 (0xC0) |
| --- | --- | --- |
| Normal | +032C | Status:SYSTEM OK |
| SDA Fault | (last temperature) | Status:SDA STUCK |
| SCL Fault | (last temperature) | Status:SCL STUCK |
| NACK Fault | (last temperature) | Status:NACK |
| Timeout | (last temperature) | Status:TIMEOUT |

## 10.3 Fault Log Clear

Pressing RB1 triggers the clear_external_eeprom() function, which resets write_index and log_count to zero and saves the new state to internal EEPROM. The external EEPROM data is not physically erased — only the counters are reset, so old records remain in EEPROM but are treated as non-existent by the log system. The LCD briefly displays a confirmation message.

# 11. Proteus Simulation

The complete system is simulated in Proteus 8 Professional. The simulation validates all fault detection, recovery, logging, and telemetry functions without physical hardware.

## 11.1 Simulation Components

| Proteus Component | Maps To | Notes |
| --- | --- | --- |
| PIC16F887 (DIL40) | Main MCU | HEX file loaded, 8MHz clock set in properties |
| LM75AD | Temperature sensor | I²C address 0x48 (A0–A2 = GND) |
| DS1307 | RTC | 32.768kHz crystal connected, VBAT tied to VCC |
| 24LC256 | External EEPROM | I²C address 0x50 (A0–A2 = GND), WP = GND |
| LM041L | HD44780 LCD | Connected to PORTD + RE0/RE1 |
| Virtual Terminal | UART monitor | Replaces COMPIM for reliable simulation |
| BUTTON | Fault injection | Connected to SDA/SCL lines via pull-up |
| 4.7kΩ PULLUP | I²C bus bias | Both SDA and SCL pulled to +5V |

## 11.2 Fault Injection Methods

| Fault | Injection Method in Proteus |
| --- | --- |
| SDA STUCK | Button between RC4 (SDA) and GND — press during simulation |
| SCL STUCK | Button between RC3 (SCL) and GND — press during simulation |
| NACK | Change address of LM75AD in schematic during runtime |
| TIMEOUT | Hold SDA or SCL button pressed for extended period |
| WATCHDOG | Inject infinite loop in code or use very long delay |

## 11.3 Simulation Debug Windows

The following Proteus debug windows are used during simulation:

- Virtual Terminal — displays all UART fault messages in real time

- PIC CPU\EEPROM Memory — shows internal EEPROM contents live during simulation

- I2C Memory\Internal Memory (U4) — shows 24LC256 external EEPROM contents

- I2C Debugger (Virtual Instruments) — shows every I²C transaction on the bus

# 12. Key Design Decisions

| Design Decision | Rationale |
| --- | --- |
| Dual-buffer EEPROM strategy | Ensures no fault is lost even if I²C bus is broken at the moment of fault |
| Magic byte first-run detection | Prevents garbage internal EEPROM values from being misread as valid log state |
| UART before I²C initialization | Prevents SSP peripheral from interfering with RC6 TX pin |
| RC6 forced HIGH in uart_init() | Ensures UART idle state is driven HIGH immediately, prevents floating TX pin |
| Watchdog Timer enabled | Provides automatic recovery from firmware deadlocks; every occurrence is logged |
| Per-step fault context recording | Allows accurate fault classification by recording which step failed and ACKSTAT state |
| 3 retry attempts before fault | Reduces false positives from transient glitches while still detecting persistent faults |
| Last known good timestamp | If RTC read fails during a fault, the last valid timestamp is used rather than 00:00:00 |
| Log count clamped at >= 256 | Handles 0xFFFF (blank EEPROM) and exact 256 boundary that > 256 would miss |
| soft clear (counters only) | Preserves physical EEPROM data while resetting the logical log, faster than byte erase |

# 13. Applications

The architecture demonstrated in this project can be applied directly to production embedded systems where communication reliability is essential:

- Industrial sensor networks — where sensors must continue operating despite bus glitches

- Automotive electronics — where safety-critical systems must log every fault for diagnostic purposes

- Smart building automation — where temperature, humidity, and pressure sensors communicate over I²C

- Medical devices — where fault logging is mandatory for regulatory compliance

- IoT edge nodes — where remote devices must self-recover without operator intervention

- Battery management systems — where cell monitoring ICs communicate over SMBus (I²C compatible)

# 14. Conclusion

This project demonstrates a complete implementation of a self-healing embedded monitoring system on the PIC16F887 microcontroller. The system successfully integrates multiple I²C peripherals with robust fault detection, automatic bus recovery, persistent logging, and real-time telemetry.

The key contribution of this design is the dual-buffer logging strategy: by using the PIC's internal EEPROM as an intermediate buffer, the system guarantees that no fault event is lost even when the I²C bus is completely non-functional. The NXP-compliant 9-pulse bus recovery procedure ensures that the vast majority of real-world stuck-bus conditions are resolved automatically without operator intervention.

The design patterns implemented here — timeout protection on every blocking operation, retry sequences before fault declaration, accurate fault classification, timestamped persistent logging, and UART telemetry — are directly applicable to production embedded systems in industrial, automotive, and IoT domains.

End of Report